EXP NO 3 :

STEPPER MOTOR USING AT89C51 USING PROTEUS

AIM:To write an assembly language program for Stepper Motor Using AT89C51 using Keil and

Proteus

SOFTWARE REQUIRED: Proteus 8 software.

PROGRAM:

ORG 0000H

UP:     MOV P2,#09H

ACALL DELAY

MOV P2,#0CH

ACALL DELAY

MOV P2,#06H

ACALL DELAY

MOV P2,#03H

ACALL DELAY

SJMP UP

DELAY:  MOV R4,#18

H1:     MOV R3,#255

H2:     DJNZ R3,H2

DJNZ R4,H1

RET

END

Circuit Diagram  :

RESULT :

Thus the program has been successfully verified and executed.