Exp No : 4

7 SEGMENT DISPLAY USING AT89C51 USING PROTEUS

AIM: To write an assembly language program for 7 Segment Display Using AT89C51 using Keil and Proteus

SOFTWARE REQUIRED: Proteus 8 software.

PROGRAM:

ORG 000H

UP:     MOV P2,#0C0H

ACALL DELAY

MOV P2,#0F9H

ACALL DELAY

MOV P2,#0A4H

ACALL DELAY

MOV P2,#0B0H

ACALL DELAY

MOV P2,#99H

ACALL DELAY

MOV P2,#92H

ACALL DELAY

MOV P2,#82H

ACALL DELAY

MOV P2,#0F8H

ACALL DELAY

MOV P2,#80H

ACALL DELAY

MOV P2,#90H

ACALL DELAY

DELAY:  MOV R5,#10

H1:     MOV R4,#180

H2:     MOV R3,#255

H3:     DJNZ R3,H3

DJNZ R4,H2

DJNZ R5,H1

RET

END

OUTPUT :

RESULT :

Thus the program has been successfully verified and executed